IMPORTANT NOTES IN MANUAL:

1- Re‐tightening: After about 10 operating hours, the coupling bolts shall be re tightened.

2- Oil check : G-LTURBO recommends having the oil analyzed after approx. 500 operating hours, and then to decide the intervals of oil change on  the basis of regular oil analyses.

3- Start by Manual Control : The compressor can be started by pressing the START button on the control panel of the local control panel. When the start button is  activated, the oil pump starts up first, followed by the main motor  when it receives the signal to start after pre‐lubricating for 1 minute.

See section on LCP.

4- Stop by Manual Control : The compressor can be stopped by pressing the STOP button on the control panel of the local control panel. When the stop button is

activated, the compressor stops normally. The auxiliary lube oil pump and the oil‐cooler ventilator will run an after‐lubrication cycle of 5 minutes.

See section on LCP.

5- Automatic  Start/Stop : Compressor units may be set in the remote‐control mode by pressing AUTO button. In this mode the compressor can start and stop without forewarning. An ISO‐7000‐0017 symbol warns of remote‐control start/stop. In this mode all guards must be correctly mounted, and no persons shall be allowed to be in the immediate vicinity of the compressor.

----------------------------------------------------------------------

PLC Power : 24 V DC – 10A

NO. Of Signals:
29 DI
20 DO
19 AI

----------------------------------------------------------------------